Handout

Comparison of Tailings Dam Construction Techniques (Table 3 in booklet)

| Embankment type | Mill tailings requirements | Discharge requirements | Water storage suitability | Raising rate restrictions | Embankment fill requirements | Seismic resistance | Relative embankment cost |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Water retention | Suitable for any type of tailings | Any discharge procedure suitable | Good | Entire embankment constructed initially | Natural soil or borrow | Good | High |
| Upstream | At least 40-60% sand in whole tailings. Low pulp density desirable to promote grainsize segregation | Peripheral discharge and well-controlled beach necessary | Not suitable for significant water storage | Less than 5m/y most desirable | Natural soil, sand tailings or mine waste | Poor in high seismic areas | Low |
| Downstream | Suitable for any type of tailings | Varies according to design details | Good | None | Sand tailings or mine waste if production rates are sufficient. Otherwise natural soil | Good | High |
| Centreline | Sands or low plasticity fines | Peripheral discharge of at least a normal beach necessary | Not recommended for permanent storage. Temporary flood storage acceptable with proper design details | Height restrictions for individual raises may apply | Sand tailings or mine waste if production rates are sufficient, otherwise natural soil | Acceptable | Moderate |