AN EXAMPLE : THE WILLIS PILOT STUDY : POTENTIAL MONITORING PROGRAM

| ROS Class | EMU | Objectives, Desired Conditions, Service Standards | Identified Issues or Threats | Indicators | Measurement |
| --- | --- | --- | --- | --- | --- |
| Roaded Natural | River & Creek Valleys | To maintain predominantly natural character of the environment To provide for family camping and associated recreational opportunities in natural setting To maintain visitor health and safety | - Weeds - Plant pathogens - Destruction of natural understory vegetation and dead timber - Soil erosion & compaction - Breaking of river banks - Proliferation of fireplaces - Littering - Quality of visitor experience - Water quality | - Extent of weed areas - Evidence of infection - Area without ground cover - Amount of natural forest floor litter - Area of bare ground - Number of sites - Number of sites - Number of incidents - Structured interviews - Standard laboratory tests to ANZEEC standards | Photography Log books to record counts and other observations Patrol at least weekly in peak season ; then quarterly Count, perhaps photograph Count (and clear) At peak times Samples from both above and below camping areas |
| Roaded natural and semi-remote motorised | Roads and Tracks | To provide safe access to area To minimise impacts upon natural environment | - Weed and pathogen introduction - Sediment fans or guttering from edge breakaways | - Number of observations - Number of occurrences | Patrols as noted above, together with special event checking after storms or floods |